COMPUTER NETWORKS LAB REPORT

Student: Gaukhar Assyrzhanova

Group: IIoT-2401

Labs: 11.5.5 Subnet an IPv4 Network | 11.9.3 VLSM Design and Implementation

Lab 11.5.5 Subnet an IPv4 Network

Part 1: Design an IPv4 Subnetting Scheme

Base network: 192.168.0.0/24. Requirements: LAN-A needs minimum 50 hosts, LAN-B needs minimum 40 hosts, plus at least 2 additional unused subnets for future expansion.

Q: How many host addresses are needed in the largest required subnet?

A: 50 this is the LAN-A requirement.

Q: What is the minimum number of subnets required?

A: 4 LAN-A + LAN-B + 2 reserved for future expansion.

Q: What is the /24 subnet mask in binary?

A: 11111111.11111111.11111111.00000000

Q: In the network mask, what do the ones represent?

A: The network portion they identify which part of the address belongs to the network.

Q: In the network mask, what do the zeros represent?

A: The host portion they identify individual devices within that network.

Subnet Mask Analysis Table

| Prefix | Decimal Mask | Subnets | Usable Hosts |
| --- | --- | --- | --- |
| /25 | 255.255.255.128 | 2 | 126 |
| /26 | 255.255.255.192 | 4 | 62 |
| /27 | 255.255.255.224 | 8 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /29 | 255.255.255.248 | 32 | 6 |
| /30 | 255.255.255.252 | 64 | 2 |

Q: Which subnet masks meet the required minimum of 50 host addresses?

A: /25 (126 hosts) and /26 (62 hosts).

Q: Which subnet masks meet the minimum number of 4 subnets?

A: /26 (4), /27 (8), /28 (16), /29 (32), /30 (64).

Q: Which subnet mask meets BOTH requirements?

A: /26 (255.255.255.192) 4 subnets with 62 usable hosts each. The most efficient choice.

Resulting Subnet Table

| Subnet Address | Prefix | Subnet Mask |
| --- | --- | --- |
| 192.168.0.0 | /26 | 255.255.255.192 |
| 192.168.0.64 | /26 | 255.255.255.192 |
| 192.168.0.128 | /26 | 255.255.255.192 |
| 192.168.0.192 | /26 | 255.255.255.192 |

Complete Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| CustomerRouter | G0/0 | 192.168.0.1 | 255.255.255.192 | N/A |
| CustomerRouter | G0/1 | 192.168.0.65 | 255.255.255.192 | N/A |
| CustomerRouter | S0/1/0 | 209.165.201.2 | 255.255.255.252 | N/A |
| LAN-A Switch | VLAN 1 | 192.168.0.2 | 255.255.255.192 | 192.168.0.1 |
| LAN-B Switch | VLAN 1 | 192.168.0.66 | 255.255.255.192 | 192.168.0.65 |
| PC-A | NIC | 192.168.0.62 | 255.255.255.192 | 192.168.0.1 |
| PC-B | NIC | 192.168.0.126 | 255.255.255.192 | 192.168.0.65 |
| ISPRouter | G0/0 | 209.165.200.225 | 255.255.255.224 | N/A |
| ISPRouter | S0/1/0 | 209.165.201.1 | 255.255.255.252 | N/A |
| ISPSwitch | VLAN 1 | 209.165.200.226 | 255.255.255.224 | 209.165.200.225 |
| ISP Workstation | NIC | 209.165.200.235 | 255.255.255.224 | 209.165.200.225 |
| ISP Server | NIC | 209.165.200.240 | 255.255.255.224 | 209.165.200.225 |

Part 2: Device Configuration

Step 1 CustomerRouter

enable

configure terminal

hostname CustomerRouter

enable secret Class123

line console 0

password Cisco123

login

exit

interface GigabitEthernet 0/0

ip address 192.168.0.1 255.255.255.192

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.0.65 255.255.255.192

no shutdown

exit

end

copy running-config startup-config

Step 2 LAN-A Switch

enable

configure terminal

interface vlan 1

ip address 192.168.0.2 255.255.255.192

no shutdown

exit

ip default-gateway 192.168.0.1

end

copy running-config startup-config

Step 3 LAN-B Switch

enable

configure terminal

interface vlan 1

ip address 192.168.0.66 255.255.255.192

no shutdown

exit

ip default-gateway 192.168.0.65

end

copy running-config startup-config

Step 4 PC-A

| Field | Value |
| --- | --- |
| IP Address | 192.168.0.62 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.0.1 |

Step 5 PC-B

| Field | Value |
| --- | --- |
| IP Address | 192.168.0.126 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.0.65 |

Part 3: Testing and Troubleshooting

Q: Can PC-A communicate with its default gateway?

A: Yes. Ping to 192.168.0.1 returns 4/4 replies, 0% loss.

Q: Can PC-B communicate with its default gateway?

A: Yes. Ping to 192.168.0.65 returns 4/4 replies, 0% loss.

Q: Can PC-A communicate with PC-B?

A: Yes. After initial ARP timeout, second attempt returns 4/4 replies, 0% loss. Full end-to-end connectivity confirmed.

Lab 11.9.3 VLSM Design and Implementation

Part 1: Examine the Network Requirements

Base network: 192.168.72.0/24. Unlike Lab 11.5.5 where all subnets use the same mask, VLSM allows each subnet to have a different size only allocating as much address space as needed.

Q: How many subnets are needed?

A: 5 subnets four LANs plus one WAN point-to-point link.

Subnet Mask Calculations

Formula: 2n − 2 ≥ required hosts, where n = number of host bits.

| LAN | Hosts Needed | Calculation | Mask | Hosts Available |
| --- | --- | --- | --- | --- |
| Sw4 | 58 | 2^6 − 2 = 62 ≥ 58 ✓ | /26 (255.255.255.192) | 62 |
| Sw3 | 29 | 2^5 − 2 = 30 ≥ 29 ✓ | /27 (255.255.255.224) | 30 |
| Sw2 | 15 | 2^4 − 2 = 14 < 15 ✗ → /27 | /27 (255.255.255.224) | 30 |
| Sw1 | 7 | 2^4 − 2 = 14 ≥ 7 ✓ | /28 (255.255.255.240) | 14 |
| WAN | 2 | 2^2 − 2 = 2 ≥ 2 ✓ | /30 (255.255.255.252) | 2 |

Note: Sw2 needs 15 hosts but /28 only provides 14 usable addresses, so /27 (30 hosts) must be used instead.

Part 2: VLSM Addressing Scheme

Subnet Table (Largest to Smallest)

| Subnet Description | Hosts | Network/CIDR | First Usable | Broadcast |
| --- | --- | --- | --- | --- |
| Sw4 LAN | 58 | 192.168.72.0/26 | 192.168.72.1 | 192.168.72.63 |
| Sw3 LAN | 29 | 192.168.72.64/27 | 192.168.72.65 | 192.168.72.95 |
| Sw2 LAN | 15 | 192.168.72.96/27 | 192.168.72.97 | 192.168.72.127 |
| Sw1 LAN | 7 | 192.168.72.128/28 | 192.168.72.129 | 192.168.72.143 |
| WAN Link | 2 | 192.168.72.144/30 | 192.168.72.145 | 192.168.72.147 |

Complete Addressing Table

Rules: Router = first usable address, Switch = second usable, PC = last usable. WAN: Remote-Site1 = first, Remote-Site2 = last.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Remote-Site1 | G0/0 | 192.168.72.129 | 255.255.255.240 | N/A |
| Remote-Site1 | G0/1 | 192.168.72.97 | 255.255.255.224 | N/A |
| Remote-Site1 | S0/0/0 | 192.168.72.145 | 255.255.255.252 | N/A |
| Remote-Site2 | G0/0 | 192.168.72.65 | 255.255.255.224 | N/A |
| Remote-Site2 | G0/1 | 192.168.72.1 | 255.255.255.192 | N/A |
| Remote-Site2 | S0/0/0 | 192.168.72.146 | 255.255.255.252 | N/A |
| Sw1 | VLAN 1 | 192.168.72.130 | 255.255.255.240 | 192.168.72.129 |
| Sw2 | VLAN 1 | 192.168.72.98 | 255.255.255.224 | 192.168.72.97 |
| Sw3 | VLAN 1 | 192.168.72.66 | 255.255.255.224 | 192.168.72.65 |
| Sw4 | VLAN 1 | 192.168.72.2 | 255.255.255.192 | 192.168.72.1 |
| User-1 | NIC | 192.168.72.142 | 255.255.255.240 | 192.168.72.129 |
| User-2 | NIC | 192.168.72.126 | 255.255.255.224 | 192.168.72.97 |
| User-3 | NIC | 192.168.72.94 | 255.255.255.224 | 192.168.72.65 |
| User-4 | NIC | 192.168.72.62 | 255.255.255.192 | 192.168.72.1 |

Part 3: Device Configuration

Step 1 Remote-Site1 Router

enable

configure terminal

interface GigabitEthernet 0/0

ip address 192.168.72.129 255.255.255.240

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.72.97 255.255.255.224

no shutdown

exit

end

copy running-config startup-config

Step 2 Sw3 Switch

enable

configure terminal

interface vlan 1

ip address 192.168.72.66 255.255.255.224

no shutdown

exit

ip default-gateway 192.168.72.65

end

copy running-config startup-config

Step 3 User-4 PC

| Field | Value |
| --- | --- |
| IP Address | 192.168.72.62 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.72.1 |

Step 4: Connectivity Verification

From Remote-Site1:

ping 192.168.72.62   → Success (User-4)

ping 192.168.72.94   → Success (User-3)

ping 192.168.72.126  → Success (User-2)

ping 192.168.72.142  → Success (User-1)

ping 192.168.72.146  → Success (Remote-Site2 WAN)

From User-4:

ping 192.168.72.1    → Success (gateway)

ping 192.168.72.65   → Success (Remote-Site2 G0/0)

ping 192.168.72.97   → Success (Remote-Site1 G0/1)

ping 192.168.72.129  → Success (Remote-Site1 G0/0)

ping 192.168.72.145  → Success (Remote-Site1 WAN)

Conclusion

In Lab 11.5.5, we applied fixed-length subnetting dividing 192.168.0.0/24 into four equal /26 subnets (62 hosts each). The /26 mask was the only one that simultaneously satisfies the 50-host minimum and provides 4 subnets (2 active + 2 reserved). In Lab 11.9.3, we applied VLSM, which is more efficient for networks with varying LAN sizes. By ordering subnets largest-to-smallest and assigning /26, /27, /27, /28, and /30 masks, we used only 148 of 256 available addresses leaving 108 for future growth. Both labs reinforced CLI configuration skills, systematic IP address planning, and ping-based connectivity verification.

End of Report —